This product has received a verified Environmental Product Declaration (EPD) based on a North American Product Category Rule (PCR.)

SECTION 08 71 13

AUTOMATIC DOOR OPERATORS

Specifier Note: This specification document shall serve as a guide specification for typical projects where the Stanley Access Technologies M-Force™ (Next Generation Magic-Force™), low energy, automatic door operator will be the basis of design. Specification must be reviewed for applicability on a per project basis. The specifier is directed to select appropriate options included herein. Consult with the local Access Technologies Territory Manager, when options, not specified, are required. See last page of this document for a summary of unspecified options.

- GENERAL

  - RELATED DOCUMENTS

    - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

  - SUMMARY

    - This Section includes the following types of automatic door operators:

      - Exterior and interior, automatic door operators, low energy, with in-ground mounting.

      - Automatic door operators shall be configured for doors as follows:

        - Simultaneous pairs.

        - Simultaneous pairs, with single operator.

        - Single doors.

    - Related Sections:

      - Division 3 Sections for concrete, grout, forming, and placing to the extent not specified in this section.

      - Division 8 Section “Doors and Frames” for entrances furnished and installed separately in Division 8 Section.

      - Division 8 Section "Aluminum-Framed Entrances and Storefronts" for entrances furnished and installed separately in Division 8 Section.

      - Division 8 Section "Door Hardware" for hardware to the extent not specified in this Section.

      - Division 26 Sections for electrical connections provided separately including conduit and wiring for power to, and control of, automatic door operators.

  - REFERENCES

    - General: Standards listed by reference, including revisions by issuing authority, form a part of this specification section to extent indicated. Standards listed are identified by issuing authority, authority abbreviation, designation number, title or other designation established by issuing authority.  Standards subsequently referenced herein are referred to by issuing authority abbreviation and standard designation.

    - Underwriters Laboratories (UL):

      - UL 325 – Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems.

    - American National Standards Institute (ANSI)/Builders’ Hardware Manufacturers Association (BHMA):

      - ANSI/BHMA A156.10: Standard for Power Operated Pedestrian Doors.

      - ANSI/BHMA A156.19: Standard for Power Assist and Low Energy Power Operated Doors.

    - American Society for Testing and Materials (ASTM):

      - ASTM A568 – Standard Specification for Steel, Sheet, Carbon, Structural, and High-Strength, Low-Alloy, Hot-Rolled and Cold-Rolled, General Requirements for.

      - ASTM A570 - Standard Specification for Structural Steel, Sheet and Strip, Carbon, Hot-Rolled.

      - ASTM B221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

      - ASTM B209 - Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

    - American Association of Automatic Door Manufacturers (AAADM):

    - National Fire Protection Association (NFPA):

      - NFPA 101 – Life Safety Code.

      - NFPA 70 – National Electric Code.

    - International Code Council (ICC):

      - IBC: International Building Code

    - Building Officials and Code Administrators International (BOCA), 1999:

    - International Standards Organization (ISO):

      - ISO 9001 - Standard for Manufacturing Quality Management Systems

  - DEFINITIONS

    - Activation Device:  Device that, when actuated, sends an electrical signal to the door operator to open the door.

    - Knowing act:	Consciously initiating the opening of a power operated door using acceptable methods including wall mounted switches such as push plates and controlled access devices such as keypads, card readers and key switches.

  - PERFORMANCE REQUIREMENTS

    - General:  Provide automatic door operators capable of withstanding loads and thermal movements based on testing manufacturer's standard units in assemblies similar to those indicated for this Project.

    - Operating Range:  Minus 30 deg F (Minus 34 deg C) to 130 deg F (54 deg C).

    - Opening-Force Requirements for Egress Doors: In the event power failure to the operator, swinging automatic entrance doors shall open with a manual force, not to exceed 30 lbf (133 N) to set door in motion, and not more than 15 lbf to fully open the door. Forces shall be applied at 1” (25 mm) from the latch edge of the door.

  - SUBMITTALS

    - General:  Submit listed submittals in accordance with Conditions of the Contract and Division 01 submittal procedures.

    - Shop Drawings:  Include plans, elevations, sections, details, hardware mounting heights, and attachments to other work. Indicate wiring for electrical supply.

    - Color Samples for selection of factory-applied color finishes.

    - Closeout Submittals: Provide the following with project close-out documents.

      - Owner’s Manual.

      - Warranties.

  - QUALITY ASSURANCE

    - Installer Qualifications:  Manufacturer's authorized representative, with certificate issued by AAADM, who is trained for installation and maintenance of units required for this Project.

    - Manufacturer Qualifications:  A qualified manufacturer with a manufacturing facility certified under ISO 9001.

    - Manufacturer shall have in place a national service dispatch center providing 24 hours a day, 7 days a week, emergency call back service.

    - Certifications: Automatic door operators shall be certified by the manufacturer to meet performance design criteria in accordance with the following standards:

      - ANSI/BHMA A156.10 and A156.19.

      - NFPA 101.

      - UL 325 Listed.

      - IBC.

      - BOCA.

    - Source Limitations:  Obtain automatic door operators through one source from a single manufacturer.

    - Product Options:  Drawings indicate sizes, profiles, and dimensional requirements of swinging doors equipped with automatic door operators and are based on the specific system indicated. Do not modify intended aesthetic effects, as judged solely by Architect, except with Architect's approval. If modifications are proposed, submit comprehensive explanatory data to Architect for review.

    - Power Operated Door Standard: ANSI/BHMA A156.19.

    - Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authorities having jurisdiction, and marked for intended use.

    - Emergency-Exit Door Requirements:  Comply with requirements of authorities having jurisdiction for swinging automatic entrance doors serving as a required means of egress.

  - PROJECT CONDITIONS

    - Field Measurements:  General Contractor shall verify openings to receive automatic door operators by field measurements before fabrication and indicate measurements on Shop Drawings.

    - Mounting Surfaces:  General Contractor shall verify all surfaces to be plumb, straight and secure; substrates to be of proper dimension and material.

    - Other trades:  General Contractor Advise of any inadequate conditions or equipment.

  - COORDINATION

    - Coordinate size and location of recesses in concrete floors for installation of operator case.  Requirements for concrete, reinforcement, grout, and formwork are specified in Division 3.

    - Templates:  Check Shop Drawings of other work to confirm that adequate provisions are made for locating and installing automatic door operators to comply with indicated requirements.

    - Electrical System Roughing-in: Coordinate layout and installation of automatic door operators with connections to, power supplies, remote activation devices, and electric door latching hardware.

    - System Integration: Integrate automatic door operators with other systems as required for a complete working installation. Where required for proper operation, provide a time delay relay to signal automatic door operator to activate only after electric lock system is released.

  - WARRANTY

    - Automatic door operators shall be free of defects in material and workmanship for a period of one (1) year from the date of substantial completion.

    - During the warranty period the Owner shall engage a factory-trained technician to perform service and affect repairs. A safety inspection shall be performed after each adjustment or repair and a completed inspection form shall be submitted to the Owner.

    - During the warranty period all warranty work, including but not limited to emergency service, shall be performed during normal working hours.

- PRODUCTS

  - AUTOMATIC DOOR OPERATORS

    - Manufacturer:

      - Stanley Access Technologies; M-Force™ Series automatic door operator.

      - In-Ground Conversion: OPCON™ Manufacturing Inc.

  - MATERIALS

    - Aluminum:  Alloy and temper recommended by manufacturer for type of use and finish indicated.

      - Headers: 6063-T6.

      - Extruded Bars, Rods, Profiles, and Tubes:  ASTM B 221.

      - Sheet and Plate:  ASTM B 209.

    - Steel:  Material recommended by manufacturer for type of use indicated.

      - Cement Case: ASTM A570, Grade A.

      - Galvanized Coating: ASTM A568.

  - COMPONENTS

    - Cement Case: Cement case shall be sheet steel formed and riveted type with galvanized coating. Cement case shall incorporate a silicone seal at each end cap and at all end cap penetrations using silicone Type RTV, specifically formulated for galvanized steel. Cement case shall incorporate a vinyl gasket at the perimeter of the cement case cover, and a shaft seal as a water and moisture seal where the spindle extends through the cement case cover and through any threshold or stone cover.

      - Cement case shall be encased below grade, utilizing setting cement. A minimum 3/8 inch (9.5 mm) of setting cement shall be placed at all exterior vertical surfaces and along the entire bottom surface. Allow setting cement to cure as required by cement product manufacturer prior to installing door panels.

      - Cement case shall be field bored to accept liquid-tight conduit connections for electric power, service, and signal wires.

    - Drive System: Drive system shall be provided to connect the operator to the transfer pivot. System shall be automatic door operator manufacturer’s approved standard chain drive with tensioner.

    - Transfer Pivot: Heavy-duty transfer pivot shall incorporate 1200 pound (544 kg) rated, sealed tapered bearings in steel bearing race, press fitted into a 1 inch (25 mm) thick aluminum bearing plate. The transfer pivot shall be mounted beneath the door leaf and jamb area utilizing a standard 2 3/4 inch (70 mm) or 3 3/4 inch (95 mm) pivot setback on center hung doors, and a 3/4 inch (19 mm) pivot setback on offset hung doors. Transfer pivot shall be adjustable horizontally and vertically plus or minus 3/8 inch (9.5 mm) minimum.

    - Bottom Door Arms: A single arm and track, designed for bottom mounting in door panel, providing positive control of door through entire swing; units shall be specifically designed for use on hinged doors as required.

    - Fasteners and Accessories:  Manufacturer's standard corrosion-resistant, non-staining, non-bleeding fasteners and accessories compatible with adjacent materials.

    - Signage:  Provide signage in accordance with ANSI/BHMA A156.19.

  - SWINGING DOOR OPERATORS

    - General:  Provide door operators of size recommended by manufacturer for door size, weight, and movement; for condition of exposure; and for long-term, maintenance-free operation under normal traffic load for type of occupancy indicated.

    - Electromechanical Operators:  Self-contained unit powered by a minimum 3/16 horsepower, permanent-magnet DC motor; through a high torque reduction gear system.

      - Operation:  Power opening and spring closing.

      - Operator Type: Low energy; readily convertible to full energy; no tools required to change type.

      - Handing: Non-handed; no tools required to change handing.

      - Capacity: Rated for door panels weighing up to 700 lb (318 kg).

      - Mounting: Visible

      - Features:

        - Adjustable opening and closing speeds.

        - Adjustable opening and closing force.

        - Adjustable back-check.

        - Adjustable hold-open time between 0 and 30 seconds.

        - Reverse on obstruction.

        - Time delay for electric lock integration.

        - Force compensation and closed loop speed control with active braking and acceleration.

        - Power Close.

        - Slam Protection.

        - Power Assist.

        - Lock Release.

        - Stall Sensor Ignore.

        - Electronic Coordination.

        - Optional Switch to open/Switch to close operation.

        - Optional push to activate operation.

        - Fire alarm interface, configurable to safely open or close doors on signal from fire alarm system.

    - Field Adjustable Spring Closing Operation: The operator shall close the door by spring energy employing the motor, as a dynamic brake to provide closing speed control. The closing spring shall be a helical compression spring, adjustable for positive closing action. The spring shall be adjustable, without removing the operator from the header, to accommodate a wide range of field conditions.

    - Independent Adjustable Closing and Latching Speed Control: The operator shall employ a rheostat module to allow for independent field adjustment of closing and latching speeds using the motor as a dynamic brake.

    - Field Adjustable Open Stop: The operator shall provide a field adjustable open stop to accommodate opening angles from 80 to 135 degrees without the need for additional components.

    - Consistent Cycle: The operator shall deliver an even, consistent open manual push force across the entire transition from door fully closed to door fully open. Additionally, the force shall be field adjustable to accommodate a wide range of on-site conditions.

    - Quiet Performance: The operator shall be designed to output audible noise ratios less than or equal to 50dba.

    - Manual Use: The operator shall function as a manual door closer in the direction of swing with or without electrical power. The operator shall deliver an even, consistent open force across the entire transition from door fully closed to door fully open.

    - Electrical service to door operators shall be provided under Division 26 Electrical.  Minimum service to be 120 VAC, 5 amps.

  - ELECTRICAL CONTROLS

    - Electrical Control System: Electrical control system shall include a microprocessor controller and a high-resolution position encoder. The encoder shall monitor revolutions of the operator shaft and send signals to microprocessor controller to define door position and speed.

      - The high-resolution encoder shall have a resolution of not less than 1024 counts per revolution. Systems utilizing external magnets and magnetic switches are not acceptable.

      - Electrical control system shall include a 24 VDC auxiliary output rated at 1 amp.

    - Performance Data: The microprocessor shall collect, and store performance data as follows:

      - Counter: A non-resettable counter to track operating cycles.

      - Event Reporting: Unit shall include non-volatile event and error recording including number of occurrences of events and errors, and cycle count of most recent events and errors.

      - LED Display: Display presenting the current operating state of the controller.

    - Controller Protection: The microprocessor controller shall incorporate the following features to ensure trouble free operation:

      - Automatic Reset Upon Power Up.

      - Main Fuse Protection.

      - Electronic Surge Protection.

      - Internal Power Supply Protection.

      - Resetable sensor supply fuse protection.

      - Motor Protection, over-current protection.

    - Power Close: When enabled, engages the operator to close a door that does not close completely at the end of a cycle.

    - Force Compensation: Utilizing the closed loop speed control, the operator shall maintain constant opening and closing speeds when subjected to excessive outside forces, such as positive or negative stack pressures.

    - Slam Protection: The operators speed control system prevents door from slamming at the full open or full closed position.

    - Power Assist: Operator mode that lowers opening forces when the door is used manually.  Power assist is active only while pushing or pulling the door.  The door will close when an opening force is no longer applied.

    - Lock Release: On doors with electric locking, operator shall include a closing function to release tension on a latch mechanism prior to opening the door.

    - Stall Sensor Ignore: Adjustable setting to disable swing side safety sensors at a specific angle.

    - Electronic Coordination: On pairs of doors, allows independent timing of opening and closing of each leaf as required for astragal coordination.

    - Soft Start/Stop: A “soft-start” “soft-stop” motor driving circuit shall be provided for smooth normal opening and recycling.

    - Obstruction Recycle: Provide system to recycle the swinging panels when an obstruction is encountered during the closing cycle.

    - Programmable Controller: Microprocessor controller shall be field programmable.

      - The following parameters may be adjusted:

        - Operating speeds and forces as required to meet specified ANSI/BHMA standard.

        - Adjustable and variable features specified.

      - Manual programming shall be available through local interface which has a two-digit display with a selection control including three push buttons.

    - Emergency Breakout Switch: A cam actuated emergency breakout switch shall be provided to disconnect power to the motor when an in-swinging door is manually pushed in the emergency out direction. The operator will then automatically reset, and power will be resumed.

    - Control Switch: Automatic door operators shall be equipped with a three-position function switch to control the operation of the door. Control switch shall provide three modes of operation, Automatic, Off, and Hold-Open.

    - Power Switch: Automatic door operators shall be equipped with a two position On/Off switch to control power to the door.

  - ACTIVATION DEVICES

    - Push Plates: Provide 4 ½ inch (114 mm) square push plates with UL recognized SPDT switch.  Face plates and mounting studs shall be stainless steel.  Face plates shall be engraved with the international symbol for accessibility and “Push To Open”. Push plates shall be wall mounted in single or double gang electrical boxes and hardwired to door operator controls.

- EXECUTION

  - INSPECTION

    - Examine conditions, with Installer present, for compliance with requirements for installation tolerances, header support, and other conditions affecting performance of swinging automatic entrance doors. Proceed with installation only after unsatisfactory conditions have been corrected.

  - INSTALLATION

    - General:  Do not install damaged components.  Fit joints to produce hairline joints free of burrs and distortion. Rigidly secure non-movement joints.

    - Cement Case:  Insert cement case in rough-in floor opening set on level bed of non-shrink, nonmetallic grout.  Fill annular space between cement case and sides of recess with non-shrink, nonmetallic grout.  Mix and place grout to comply with grout manufacturer's written instructions.

    - Mounting:  Install automatic door operators plumb and true. Anchor securely in place. Set arms and linkages level and true to location with anchorage for permanent support.

    - Door Operators:  Connect door operators to electrical power distribution system as specified in Division 26 Sections.

  - FIELD QUALITY CONTROL

    - Testing Services:  Factory Trained Installer shall test and inspect each swinging automatic entrance door to determine compliance of installed systems with applicable ANSI standards.

  - ADJUSTING

    - Adjust door operators, controls, and hardware for smooth and safe operation, for tight closure, and complying with requirements in ANSI/BHMA A156.19 by AAADM Certified Technician.

  - CLEANING AND PROTECTION

    - Clean surfaces promptly after installation.  Remove excess sealant compounds, dirt, and other substances.  Repair damaged finish to match original finish.

END OF SECTION 08 71 13

Available options not specified in this document are summarized as follows:

Activation options (Standard Hard Wired Push Plates Specified) including:

Wireless push plates.

Wireless transmitters.

Push plate posts.

Push to operate.

Safety options (None Specified) including:

Overhead safety systems.

Door mounted safety systems.

Contact your local Stanley Access Technologies representative for more information on specifying the right sliding automatic entrance for your project.

These specifications represent a “sample” configuration and depict design features that are commonly used. These specifications do not reflect “standard” features and are provided for informational purposes only. Please note that there is no standard “off the shelf” product.  Stanley custom manufactures each product to its customers’ specifications. It is the customer’s responsibility to validate that a particular configuration of Stanley’s products is suitable for a specific application.  All specifications and designs contained herein are subject to change without notice or obligation.